Arduino Measuring Tape

Bare-Minimum Prototype — Setup Plan

Version 0.1.0  ·  14 October 2026

Target: a hand-held node that streams live distance readings to an Android phone over classic Bluetooth SPP. This document is the complete, step-by-step plan to build and verify that prototype.

# 1. Purpose & definition of done

Point the sensor at something and see a live number on the phone, updating about five times per second over Bluetooth.

The prototype is considered working when:

The serial monitor shows DIST: frames at ~5 Hz and the value tracks a moving hand.

A generic Android Bluetooth serial terminal receives the same DIST: frames.

The Expo app displays the live value on its Main screen within ~500 ms of a real frame.

This is a spike, not a shippable build. It is USB-powered, assembled on a breadboard (no soldering), hardcoded to a single already-bonded device, and has no persistence or automatic reconnect.

# 2. Scope

## In scope

Flash the existing firmware and prove the radio link.

Add react-native-bluetooth-classic, its config plugin and Android permissions.

Write a minimal SppTransport behind the existing DeviceTransport contract.

Drive the app state from the transport and show live values in the existing screens.

One connect / disconnect control.

## Out of scope (accepted gaps)

Persistence, auto-reconnect with backoff, pushing THRESH:/BEEP: on connect, periodic PING, alert-sound playback, temperature compensation, datum offset, battery/boost/charger, enclosure and cutouts, parser unit tests. If the Bluetooth link drops, restart the app.

# 3. Bill of materials

Buy these; confirm you already own the UNO and HC-SR04.

| Item | Qty | Notes |
| --- | --- | --- |
| HC-05 (or HC-06) Bluetooth SPP module | 1 | Buy the 6-pin base-board variant with header already soldered. |
| HC-SR04 ultrasonic sensor | 1 | Confirm you have one. |
| Arduino UNO R3 | 1 | Uses a USB-B port. |
| Resistor 1 kΩ | 1 | HC-05 RX divider, series leg. |
| Resistor 2 kΩ | 1 | HC-05 RX divider, to GND. |
| Piezo buzzer (passive) | 1 | Firmware drives it with tone(); buy one with pins. |
| Resistor 220 Ω | 1 | Buzzer series resistor. |
| Breadboard (830-point) | 1 | No soldering for the prototype. |
| Jumper wires (M–M and M–F) | ~20 | HC-05 pins are male; the sensor uses female headers. |
| USB-A to USB-B cable | 1 | Flashes and powers the UNO. |

Deferred to later milestones (not needed to work): Li-ion cell, TP4056 / charge-boost module, power switch, project box, M3 standoffs, perfboard, bulk/decoupling capacitors, soldering iron. The essentials total roughly $15–30 if the UNO and HC-SR04 are already on hand.

Warning: do not skip the divider. Put 1 kΩ in series and 2 kΩ to GND on the UNO TX (D3) to HC-05 RX line. Direct 5 V into a 3.3 V input degrades the module over time.

# 4. Pin map

Fixed by the existing firmware sketch.

| UNO pin | Connects to | Notes |
| --- | --- | --- |
| D9 | HC-SR04 TRIG | Output pulse. |
| D10 | HC-SR04 ECHO | Input, 5 V — direct OK. |
| D8 | 220 Ω → buzzer + | Buzzer − to GND. |
| D2 | HC-05 TXD | SoftwareSerial RX. |
| D3 | 1 kΩ → HC-05 RXD | SoftwareSerial TX, via divider. |
| 5V | HC-SR04 VCC, HC-05 VCC | Base board tolerates 5 V. |
| GND | Sensor GND, HC-05 GND, buzzer −, divider lower leg | Common ground. |

Leave KEY and STATE on the HC-05 unconnected. KEY unconnected keeps the module in normal data mode; STATE is only for a future connection LED.

# 5. Wiring

Schematic:

+5V --------- ------+--------------+------ +5V rail
                                            |              |
                                        HC-SR04 VCC     HC-05 VCC
                                            |              |
  Arduino UNO                               |              |
  +----------+   D9  ---------------------- > TRIG         |
  |          |   D10 <----------------------- ECHO         |
  |          |   D2  <------------------------------------- TXD  HC-05
  |          |   D3  --[ 1k ]--+---------------------------- RXD  HC-05
  |          |                 |
  |          |               [ 2k ]
  |          |                 |
  |          |   D8  --[ 220R ]+--> buzzer +     (buzzer - -> GND)
  |          |   GND ------------------------------------- GND rail
  +----------+

Divider maths: a 1 kΩ series resistor from D3 and 2 kΩ to GND at the RXD node gives 5 V x 2/(1+2) ≈ 3.33 V — within the HC-05's 3.3 V input.

## Build order

| Step | Action | Check |
| --- | --- | --- |
| W1 | Link the breadboard power rails to the UNO: red rail → 5V, blue rail → GND. | Multimeter: ~5.0 V rail-to-rail. |
| W2 | Seat the HC-SR04 across the centre trench; wire VCC, GND, TRIG→D9, ECHO→D10. | Tug-test jumpers; no crossed legs. |
| W3 | Seat the HC-05; wire VCC→5V, GND→GND, TXD→D2. Do not wire RXD yet. | HC-05 LED blinks ~1 Hz (not paired). |
| W4 | Build the divider: 1 kΩ in series, 2 kΩ to GND, tap → HC-05 RXD; other end → D3. | See W5 before enabling TX. |
| W5 | With the UNO powered and idle, measure the RXD node to GND. | Must read ≈3.3 V (<3.6 V). If 5 V, the divider is wrong. |
| W6 | Wire the buzzer: D8 → 220 Ω → buzzer +; buzzer − → GND rail. | — |
| W7 | Continuity sweep: no short between the 5 V and GND rails. | Multimeter: no beep. |

Complete W1–W6 before the first full power-up. The W5 check is the one that protects the HC-05.

# 6. Firmware setup

No firmware changes are required. The existing sketch already samples at 5 Hz, applies EMA smoothing, outputs DIST: / DIST:ERR, mirrors to USB serial, and handles PING, THRESH: and BEEP:.

Open firmware/measuring_tape/measuring_tape.ino in the Arduino IDE (or use arduino-cli).

Select board "Arduino UNO".

Upload over USB.

# 7. App setup

This is the only real new code. It is additive because the screens already depend on the DeviceTransport types.

| ID | Task | Deliverable |
| --- | --- | --- |
| PB-1 | npx expo install react-native-bluetooth-classic; add the with-rn-bluetooth-classic plugin and Android permissions in mobile/app.json (BLUETOOTH_CONNECT/SCAN for API 31+, ACCESS_FINE_LOCATION for ≤30). | Configured project |
| PB-2 | Build a dev build: npx expo run:android (the module does not run in Expo Go). | Dev build on the phone |
| PB-3 | Implement SppTransport in mobile/src/services/sppTransport.ts: connect() to a target device; subscribe() buffers bytes, splits on newline, parses DIST:<cm> (skip DIST:ERR). | Transport module |
| PB-4 | Target a device via a DEVICE_MAC / name constant in mobile/src/config.ts (assume already bonded in Android Settings). | Device target |
| PB-5 | Wire the transport into mobile/src/state/AppState.tsx; set connection from the transport and flip SIMULATE_LIVE_DATA = false. | Live readings in the UI |
| PB-6 | Add a minimal connect/disconnect control plus the existing status badge reflecting connection. | Connection UI |

Files touched:

| File | Change |
| --- | --- |
| mobile/app.json | Bluetooth config plugin + Android permissions. |
| mobile/package.json | react-native-bluetooth-classic dependency. |
| mobile/src/services/sppTransport.ts | New — the core work. |
| mobile/src/services/device.ts | Minor — device target / optional connection-state listener. |
| mobile/src/config.ts | SIMULATE_LIVE_DATA = false; device id constant. |
| mobile/src/state/AppState.tsx | Consume the transport instead of the simulator. |
| one screen (Main or Settings) | Connect / disconnect control. |

# 8. Verification ladder (cheapest first)

Serial only — flash the UNO and open the Arduino monitor at 9600 baud; confirm DIST: streams and tracks a hand.

Command sanity — send PING (expect PONG), THRESH:20 (expect OK:THRESH=20.0), BEEP:0 (expect OK:BEEP=0 and silence).

Radio — pair the HC-05 and read DIST: in a generic Android serial terminal (proves radio + divider + baud, no app code).

App — connect and watch the hero value track the sensor within ~500 ms.

# 9. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| No DIST: on serial | One-way TX/RX or sensor unpowered | Recheck D9/D10 and sensor VCC/GND. |
| Serial fine, Bluetooth silent | TXD/RXD swapped | HC-05 TXD → D2; UNO D3 → RXD (via divider). |
| Garbled data over BT | Baud or divider wrong | Confirm RXD node ≈3.3 V; module at 9600. |
| HC-05 LED stays dark | Wrong VCC/GND or dead module | Confirm 5 V on base-board VCC. |
| Cannot pair | Module in an odd state | Power-cycle; confirm a data-mode default. |
| Need a different baud/name | Default config | Use KEY (or the base-board button) for AT mode. |

# 10. Risks

| Risk | Mitigation |
| --- | --- |
| react-native-bluetooth-classic may lag RN 0.86 / the new architecture. | Do the generic-terminal test first; fall back to an HM-10 / BLE transport if the build breaks. |
| HC-05 RX damaged by 5 V. | Fit the 1 kΩ + 2 kΩ divider and verify ≈3.3 V before connecting. |
| Classic SPP varies across Android OEMs and never works in emulators. | Test on a physical device only; declare all permissions. |
| Split serial packets. | Line-buffer in SppTransport (accumulate until newline). |

# 11. Immediate next steps

Order the HC-05 (6-pin base board) and divider resistors.

Breadboard and wire the circuit (section 5), validating the divider at W5.

Flash the sketch and pass verification steps 1–3.

Add the Bluetooth module, build a dev build, and implement SppTransport.

Run verification step 4 — live value on the phone.

Arduino Measuring Tape — Bare-Minimum Prototype Setup Plan · v0.1.0 · See docs/PROTOTYPE-PLAN.md, docs/PROTOTYPE-SHOPPING-LIST.md and the prototype wiring plan for the markdown sources.